Our Project — Full Details in Simple Words

AI Noise Cancellation for Defence Communications — Team Phoenix

| Problem Statement SIH 2026 - PS 26052 | Team Phoenix |
| --- | --- |
| Who's it for DRDO / IDEX | Runs on Raspberry Pi (small offline computer) |

# 1. What We Built

We built a small AI program that listens to someone talking through a microphone, and in real time, decides which parts of the sound are their voice and which parts are unwanted noise — gunfire, explosions, vehicle engines, people talking in the background, equipment sounds. It then quiets down the noisy parts while leaving the voice alone, so what comes out sounds clearer. The whole thing runs on a small, cheap computer called a Raspberry Pi, and it never needs an internet connection to work — which matters a lot for defence use, since you can't count on WiFi being available out in the field, and voice data shouldn't be sent over a network anyway.

# 2. The Problem We're Solving

When people talk over radios or phones in noisy environments — battlefields, vehicles, crowded areas — background noise makes it hard to hear and understand them. This can slow things down or cause dangerous misunderstandings. Two common fixes already exist, but both fall short:

- Noise-cancelling headphones work by physically playing an opposite sound wave to cancel out noise. This works well for steady sounds like engine hum, but it's much weaker against sudden loud sounds like a gunshot or explosion, which is exactly the kind of noise that matters most here.

- Cloud-based AI noise cleanup apps (the kind used in video calls) need a live internet connection to work, since the audio is processed on a remote server, not on your own device. That's a non-starter for defence use — you can't depend on network access in the field, and sending voice data over a network at all is a security risk.

So the goal was: build something AI-smart enough to tell voice apart from noise, including sudden loud noise, but that runs completely on its own, on cheap hardware, with nothing ever leaving the device.

# 3. How It Works, Step by Step

## 3.1 Breaking the sound into tiny pieces

The microphone's audio is chopped into very short pieces — just 16 milliseconds each, so about 60 pieces happen every single second. Each little piece is then split into different "pitch bands" (frequency ranges), a bit like separating a musical chord into its individual notes. Some pitch bands usually carry more of a person's voice, others usually carry more noise — this splitting is what lets the AI examine the sound in enough detail to tell the two apart.

## 3.2 The AI "brain" that decides what's noise

For every tiny piece of sound, a small AI model looks at it — along with the last 8 pieces just before it (a little over one-tenth of a second of recent history) — and, for every pitch band, predicts a number from 0 to 1: 0 means "this is basically all noise, mute it," 1 means "this is basically all voice, keep it as-is," and anything in between is a partial turn-down. We deliberately used a small, simple AI design rather than a large, complex one, for two practical reasons: a small model runs fast enough on cheap hardware with no lag, and it only ever looks backward at sound that already happened — it never needs to "wait and see what comes next," which keeps the delay it adds to a conversation essentially unnoticeable.

## 3.3 How we taught it to tell voice from noise

The AI learned by example, not by being told rules. We took clean recordings of people talking and digitally mixed them with real noise recordings at different loudness levels, so the AI always had a mixed (noisy) version to look at and a clean version to compare its guess against. Over many thousands of these examples, it gradually got better at guessing which parts of a mixed sound were voice and which were noise. Importantly, we specifically recorded and added our own noise clips — gunshots, an explosion sound, hand claps, and keyboard/equipment typing — on top of more generic noise, so the AI has direct experience with the kinds of sudden, tactical-relevant sounds this project actually needs to handle, not just everyday background noise like fans or traffic.

## 3.4 Making the cleaned-up voice sound natural

Here's a subtlety: removing noise can only ever make the sound quieter, never louder — even when it's working correctly. Left alone, that would make the cleaned-up voice sound weaker than it should, which isn't what you want for actual communication. So we added a volume top-up step that automatically brings the loudness back up to a natural level after the noise is removed, so a suppressed, clean voice doesn't sound artificially faint. We also built in an adjustable "how aggressive" dial for the suppression itself, so the strength can be tuned up or down by ear based on real listening tests, without needing to retrain the AI from scratch every time.

## 3.5 Getting it running on the Raspberry Pi

The AI was originally trained using PyTorch, a common but fairly heavy piece of training software, on a laptop. That's fine for training, but installing all of that on a small, low-power Raspberry Pi isn't necessary once training is done — so we converted the trained AI into a lighter, more portable format called ONNX, which a much smaller and simpler program can run directly. We double-checked that this conversion didn't change the AI's behaviour at all — its answers before and after converting matched to within a tiny fraction of a percent, so nothing was lost in translation. On the Pi, everything then runs fully offline: audio comes in over a USB microphone, gets processed on the spot, and plays back out — no server, no network call, nothing leaves the device at any point.

## 3.6 Doing all of this live, not after the fact

This isn't a "clean up a recording afterward" tool — it processes audio continuously, in tiny live pieces, the whole time someone is speaking. The exact same step-by-step process was first proven working on a laptop with a live microphone before being moved onto the Raspberry Pi, so the real-time behaviour was tested and confirmed before we ever relied on it running on the smaller hardware.

# 4. What We Actually Tested and Measured

- Speed: on the Raspberry Pi, the AI takes about 0.27 milliseconds on average to process each tiny sound piece — and each piece is allowed up to 16 milliseconds before it would start causing a delay. That means it finishes over 55 times faster than it actually needs to, so there's a huge safety margin and it never lags, even worst-case.

- Size: the trained AI itself is only about 310 KB once converted — smaller than a typical photo — so it's light to store and quick to load.

- Real noise reduction: when we played a loud noise (like a gunshot) close to the microphone while someone talked, the cleaned-up output measured roughly 50-90% quieter specifically at the exact moments the loud noise happened, compared to the raw, unprocessed input. Just as importantly, that big drop only happens around the noisy moments, not evenly across the whole recording — which shows the AI is actually targeting the noise itself, not just turning the whole volume down blindly.

- Whole pipeline tested end-to-end: before ever trying this live, we tested every stage in order — training the AI, converting it to the lighter format, copying it onto the Raspberry Pi, and confirming it runs correctly and fast enough there — so each step was verified working before building on top of it.

# 5. Why We Made These Choices

A few decisions came up a lot while building this, and it's worth being able to explain the reasoning behind each one simply:

| What we chose | Why, in plain words |
| --- | --- |
| Working pitch-by-pitch, not on the raw sound wave | Splitting sound into pitch bands and deciding voice-vs-noise per band is a lighter, faster calculation than trying to work with the raw waveform directly — and it's a well-proven approach used by other lightweight noise-suppression tools. |
| A small, simple AI instead of a big, powerful one | Bigger AI models generally need more computing power than a tiny Raspberry Pi has, and many need to "look ahead" at audio that hasn't happened yet to work well, which adds delay. A small model avoids both problems. |
| Running fully on the device, not sending audio to the cloud | No internet needed at all, and nothing about a person's voice ever leaves the device — both are essential for a defence communications tool, where you can't rely on network access and shouldn't be transmitting voice data over a network unnecessarily. |
| Converting to the lighter "ONNX" format instead of running the original training software on the Pi | The original training software is much bigger and heavier than needed just to run an already-trained AI. The lighter format does the same job with a far smaller footprint, which suits a small, low-power computer. |
| An adjustable suppression strength instead of one fixed setting | Lets the team fine-tune how aggressively noise gets removed based on how it actually sounds in real listening tests, rather than being locked into a single guess made ahead of time. |

# 6. What's Not Done Yet (Being Honest About It)

It lands better with a technical audience if these come from us first, with a plan, rather than being caught off guard:

- Needs a USB microphone or headset: the Raspberry Pi's own built-in headphone port cannot accept a microphone at all, on any Raspberry Pi model — it only plays sound out, it can never listen through that port.

- Not formally tested on totally unfamiliar noise: we expect it to handle noise sounds it wasn't specifically trained on reasonably well, since it's not memorizing exact sounds, just learning general voice-vs-noise patterns — but we haven't measured this with real numbers yet.

- Not tested with multiple people talking at once: the current design is built and tested around one main speaker plus background noise.

- Not yet ruggedized for real battlefield conditions: this is a working, tested prototype on regular hardware — not yet built into a dust-proof, shock-proof, weatherproof unit, and not yet through any official military testing.

TEAM TO FILL IN: the exact hardware cost, a measured power/battery draw number, and the plan for physically connecting this to real radio equipment — these depend on choices only your team has made and aren't covered here.

# 7. Questions You Might Get Asked, With Simple Answers

Every question here is answered honestly and in plain words, grounded in what's actually been built and tested — nothing made up.

## 7.1 About how it actually works

Q: How does it know what's voice and what's noise?

A: It doesn't recognize specific words — it works on the sound's pitch content. For every tiny slice of audio, it looks at each pitch band and predicts how much of it is probably voice versus probably noise, based on patterns it learned from thousands of training examples of mixed voice-and-noise audio.

Q: Why use a small, simple AI instead of a big, powerful one?

A: Because of where it needs to run: a small, cheap Raspberry Pi, with no lag allowed. Bigger AI models usually need more computing power than that, and many also need to look ahead at audio that hasn't happened yet, which adds delay. Our small model only looks backward, so it stays both fast and truly real-time.

Q: How much delay does it add — would someone notice a lag while talking?

A: The processing happens in 16-millisecond pieces, and the AI itself finishes its part in about 0.27 milliseconds — more than 55 times faster than it needs to. We haven't separately measured the very last bit of delay from the audio hardware itself yet, but the AI is clearly not the bottleneck.

Q: How did you train it, and what data did you use?

A: By mixing clean voice recordings with real noise recordings at different loudness levels, and having the AI learn to guess, for each mixed example, what the original clean voice must have sounded like. We added our own recordings of gunshots, an explosion, claps, and typing noise specifically so it has direct experience with defence-relevant sounds, not just everyday noise.

Q: Why convert the AI into this "ONNX" format instead of just running it as originally trained?

A: The original training software is heavy and not meant to be installed on small, low-power devices. Converting to a lighter, portable format lets the Raspberry Pi run the same AI with far less software overhead. We checked that the converted version behaves identically to the original — the difference in its answers was tiny enough to be meaningless.

Q: Could it accidentally mute someone's real voice?

A: It's possible, like with any AI-based system, which is why we built in an adjustable strength dial rather than one fixed hardcoded setting — so it can be tuned by ear. In our own tests, voices stayed clearly understandable even while loud background noise was cut by 70-90%.

Q: What happens if it hears a noise it's never heard in training?

A: Because it works on general sound-pattern differences rather than recognizing a fixed list of specific noises, we expect it to still handle unfamiliar noise reasonably, with performance gradually getting less certain rather than suddenly failing. That said, we haven't formally tested this with real numbers against a set of totally new noise types yet — that's honest, ongoing work, not a finished claim.

## 7.2 About whether this actually works in real life

Q: What hardware do you actually need to run this?

A: A Raspberry Pi (or a similar small, cheap computer) plus a USB microphone or headset — no powerful computer, no special graphics hardware, and no internet connection required. A regular wired microphone can't plug directly into the Pi's own audio port, since that port can only play sound out, never listen.

TEAM TO FILL IN: the exact cost of the specific Pi model and USB audio hardware your team used, since it wasn't tracked as part of the AI/software work.

Q: Does it need internet or a network connection to work?

A: No — once it's set up, the entire process (listening, deciding what's noise, cleaning it up, playing it back) happens fully on the device itself, with zero network calls. We only ever used a network connection during our own development work (like transferring files while building it), never as part of how the finished system actually runs.

Q: How much power or battery does it use?

A: Small computers like the Raspberry Pi are generally designed to use much less power than a full-size computer, and are commonly run on battery packs in similar projects — but we haven't yet measured our specific setup's actual power draw while it's running.

TEAM TO FILL IN: an actual measured power draw number (a small USB power meter would do it) before presenting — quick to get, and useful to have ready.

Q: Could you scale this up to equip a lot of people, not just one prototype?

A: Yes, fairly easily on the software side — the trained AI is a single small file (about 310 KB) that can simply be copied onto as many devices as needed, with no per-device cloud cost or central server involved, since every unit runs completely independently. Scaling the physical hardware side follows normal supply and manufacturing processes, separate from the AI itself.

Q: How is this different from noise-cancelling headphones, or apps like Krisp?

A: Noise-cancelling headphones work physically, by playing an opposing sound wave, and are mainly effective against steady sounds like engine hum — much less effective against a sudden, sharp sound like a gunshot. Apps like Krisp use a similar AI idea to ours, but they're generally built for everyday call noise (typing, fans, traffic) and usually rely on cloud processing over a network. Ours is tuned specifically for tactical, defence-relevant noise, and runs fully offline on the device itself.

## 7.3 Is it ready, and what's left

Q: Is this actually ready to use in the field today?

A: The core AI and software — training it, running it live, and getting it working on the Raspberry Pi — is built and genuinely tested, with real measured numbers behind it. What's still missing before real field use is making the hardware rugged (dust-proof, shock-proof, weather-resistant casing) and putting it through official testing — we're upfront that this is a working prototype, not a finished, field-hardened product yet.

Q: How would this actually connect to real radios or communications equipment?

A: It takes in microphone audio and outputs cleaned-up audio over a standard USB audio connection, so conceptually it would sit between a person's headset/microphone and their radio, cleaning the voice before it's transmitted. Exactly how it would be wired into a specific radio system is a hardware-integration step beyond what we've built so far.

Q: What would you focus on next, with more time?

A: Formally testing how well it handles totally new noise it hasn't seen before, measuring the true end-to-end delay and actual power usage, testing it with more than one person talking at once, and moving from this working prototype toward a properly ruggedized unit that could plug into real radio hardware. We'd rather name these directly than pretend everything is already finished.